INSTALLATION GUIDE FOR NON-ILLUMINATED CHANNEL LETTERS

Installing Acrylic, PVC, or Stainless Steel dimensional letters is simple and efficient when using the supplied installation template. Follow the steps below for a professional result.

━━━━━━━━━━━━━━━━━━━━

PRE-INSTALLATION CHECKLIST

Included

1:1 Scale Installation Template

Required Tools

Laser Level

Power Drill

Painter's Tape

Microfiber Cloth

Neutral Silicone Sealant

Mounting Hardware

Threaded Studs

Wall Anchors

Spacer Sleeves (Optional)

━━━━━━━━━━━━━━━━━━━━

STEP 1

SURFACE INSPECTION

Ensure the mounting surface is clean, dry, and structurally stable before installation.

Concrete / Brick

Use masonry drill bits and expansion anchors.

Drywall / Wood

Use wood screws or toggle bolts.

Metal Panels

Use self-tapping screws or rivets.

Glass / Stone

Use heavy-duty VHB tape and neutral silicone adhesive.

━━━━━━━━━━━━━━━━━━━━

STEP 2

POSITION THE TEMPLATE

Place the supplied 1:1 scale paper template on the wall.

Use a laser level to align the baseline accurately.

Secure all corners with painter's tape to prevent movement during installation.

━━━━━━━━━━━━━━━━━━━━

STEP 3

PRECISION DRILLING

Mark all fixing locations according to the template.

Drill holes to match the required stud depth.

Remove all dust and debris from the holes before mounting.

Clean holes ensure maximum holding strength.

━━━━━━━━━━━━━━━━━━━━

STEP 4

MOUNT THE LETTERS

Flush Mount

Install threaded studs into the back of each letter.

Apply neutral silicone into the wall holes.

Insert the studs and press the letters firmly against the wall.

Standoff Mount

Install spacer sleeves over the studs before mounting.

This creates a premium floating appearance and enhanced dimensional effect.

━━━━━━━━━━━━━━━━━━━━

STEP 5

WEATHERPROOFING & CLEANING

Apply outdoor-grade neutral silicone around mounting points where required.

Remove the installation template carefully.

Clean the letter surfaces using a microfiber cloth.

Remove fingerprints, dust, and installation residue.

━━━━━━━━━━━━━━━━━━━━

STEP 6

FINAL INSPECTION

Inspect the sign from a viewing distance of 5–10 meters.

Check:

Alignment

Letter Spacing

Mounting Stability

Overall Appearance

Ensure all letters are secure and properly positioned before project completion.

━━━━━━━━━━━━━━━━━━━━

IMPORTANT INSTALLATION NOTES

Keep the protective film on Acrylic and PVC letters until installation is complete.

Wear clean cotton gloves when handling Stainless Steel letters.

Use only neutral-cure outdoor silicone.

Do not use acidic adhesives or instant glues.

For large Acrylic or PVC signs, allow slight clearance around mounting holes to accommodate thermal expansion and contraction.