Cabinet Sign Installation Guide

Scope of Application

This guide applies to wall-mounted, pole-mounted, pylon, hanging, blade and monument cabinet signs. Approved installation drawings, manufacturer instructions and local building and electrical codes shall take priority.

Non-illuminated cabinet signs can follow the same structural steps while omitting the electrical procedures.

1. Pre-Installation Preparation

- Survey the site and verify the wall, pole, foundation or supporting structure.

- Confirm the cabinet dimensions, total weight, mounting method and wind-load requirements.

- Check the cabinet, sign faces, brackets, fasteners, LED drivers, wiring and accessories.

- Confirm the power supply, conduit, junction box and grounding arrangement.

- Locate hidden cables, pipes and underground utilities before drilling.

- Prepare suitable PPE, access equipment and rated lifting equipment.

2. Mark the Installation Position

- Mark the sign centerline, installation height and fixing points.

- Use the mounting drawing or installation template to maintain correct alignment.

- Confirm that fixing points connect to solid masonry, structural framing, supporting steel or an approved foundation.

- Allow enough space for sign-face removal and future maintenance.

3. Install the Supporting Structure

| Mounting Type | Installation Requirement |
| --- | --- |
| Wall-Mounted | Fix brackets or rails to concrete, masonry, structural studs or blocking. Do not rely on decorative cladding alone. |
| Pole-Mounted / Pylon | Confirm the poles, crossbars, anchor bolts and foundation comply with the structural drawing. |
| Hanging / Blade | Secure the projecting bracket to the building structure and add a safety restraint where required. |
| Monument | Align the cabinet with the foundation, base plate or embedded support frame. |

Use anchors and fasteners suitable for the substrate, cabinet weight and calculated wind load. Check that the complete support is level, plumb and securely tightened.

4. Lift and Secure the Cabinet

- Secure removable sign faces and access panels before lifting.

- Attach rated slings only to the designated lifting points.

- Lift the cabinet slowly and align it with the brackets or support frame.

- Install all fixing bolts and tighten them gradually to avoid cabinet deformation.

- Recheck the cabinet for level and plumb.

- Use corrosion-resistant shims where adjustment is required.

5. Complete the Electrical Connection

Electrical work must be completed by qualified personnel.

- Switch off, isolate and verify the power supply before wiring.

- Confirm that the supply voltage matches the sign and LED driver specifications.

- Route the power cable through weatherproof conduit to the designated cable entry.

- Connect the branch circuit to the disconnect, junction box or LED driver input according to the wiring diagram.

- For multi-section cabinets, connect the labeled wiring harnesses and confirm polarity.

- Ground and bond the metal cabinet according to local electrical requirements.

- Seal cable entries with weather-rated fittings or compatible sealant.

Factory-wired LED modules should not be modified unless instructed by the manufacturer.

6. Install the Sign Face and Complete Weather Sealing

- Install or refit the acrylic, polycarbonate, metal or flex sign face.

- Secure all retainers, hinges, gaskets and access covers.

- Do not over-tighten rigid acrylic or polycarbonate panels.

- Tension flex-face material gradually and evenly to prevent wrinkles.

- Apply exterior-grade sealant around cable entries and fixing penetrations.

- Do not block drainage holes, ventilation openings or service-access points.

7. Function Test and Final Inspection

- Restore power and check for flickering, dark areas or uneven illumination.

- Test both faces of a double-sided cabinet sign.

- Test dimming, RGB control, timers or photocells when included.

- Inspect all brackets, bolts, sign faces, gaskets and cable entries.

- Confirm that drainage and ventilation openings remain clear.

- Clean the cabinet and sign face after installation.

- Provide the customer with wiring diagrams and maintenance instructions.

Important Safety Notes

- Follow approved structural drawings and local installation regulations.

- Use fasteners suitable for the cabinet weight and wind load.

- Do not install during rain, lightning or strong winds.

- Keep personnel clear of suspended loads.

- Use lifting equipment rated for the complete cabinet weight.

- Do not drill into unapproved cabinet locations.

- Proper grounding is required for illuminated metal cabinet signs.

- Do not block drainage, ventilation or maintenance openings.